# BT GATT Protocol Specification BQ Device

BT GATT Protocol Specification - BQ Device

Overview

This document defines the Bluetooth Low Energy (BLE) GATT services and characteristics for the BQ pill dispenser device. Uses the BLEOTA library for OTA firmware updates.

Custom services use 128-bit UUIDs with the base format: XXXXXXXX-0000-1000-8000-00805F9B34FB.

Service Summary

| Service Name | Service UUID | Description |
| --- | --- | --- |
| Device Information (DIS) | 0x180A | Device identification (standard BLE DIS) |
| BLE OTA | 00008018-0000-1000-8000-00805f9b34fb | Firmware update via BLEOTA protocol |
| Time Sync | 0x1802 | Device clock synchronization |
| Terminal Command | 0x1803 | Debug/diagnostic terminal |
| Device Sensors | 0x1804 | Sensor status monitoring |
| Device Control | 0x1805 | Movement and control commands |
| Extract Pill | 0x1806 | Pill extraction operations |
| Sub Drive | 0x1807 | Sub drive information |
| Notifications | 0x1808 | Event notifications |
| Offline Sync | 0x1809 | Offline data synchronization |
| Battery | 0x180F | Battery level (standard BLE) |
| Sound File | 0x180B | Audio file management |

1. Device Information Service (DIS)

Service UUID:0x180A (Standard BLE DIS)

Provides device identification and status information. Uses the standard Bluetooth Device Information Service combined with BLEOTA library methods.

Characteristics

| Characteristic | UUID | Properties | Description | API Method |
| --- | --- | --- | --- | --- |
| Model Number | 0x2A24 | Read | Device model string | setModel() |
| Serial Number | 0x2A25 | Read | Device serial number | setSerialNumber() |
| Firmware Revision | 0x2A26 | Read | Firmware version string | setFWVersion() |
| Hardware Revision | 0x2A27 | Read | Hardware revision string | setHWVersion() |
| Manufacturer Name | 0x2A29 | Read | Manufacturer identifier | setManufactuer() |

Battery Service

Battery level is provided via a separate standard Battery Service (0x180F):

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Battery Level | 0x2A19 | Read, Notify | Current battery percentage (0-100) |

Battery Level Format

Byte 0: Battery percentage (0x00 - 0x64)

Firmware Version Format

Bytes 0-N: Major.Minor.Patch as ASCII string (null-terminated)
Example: "1.2.3"

2. BLE OTA Service (BLEOTA)

Service UUID:00008018-0000-1000-8000-00805f9b34fb

Reference:github.com/gb88/BLEOTA

Handles over-the-air (OTA) firmware and SPIFFS filesystem updates using the BLEOTA protocol. Supports zlib compression (~30% faster transfers) and RSA-2048 signature verification.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Firmware Receive | 00008020-0000-1000-8000-00805f9b34fb | Write, WriteNR, Notify | Firmware data reception with ACK |
| Command | 00008022-0000-1000-8000-00805f9b34fb | Write, WriteNR, Notify | OTA commands and responses |

Command Packet Format

Bytes 0-1: Command ID (uint16, little-endian)
Bytes 2-17: Payload (16 bytes, zero-padded if unused)
Bytes 18-19: CRC16 checksum

Total: 20 bytes

Command Codes

| Command | ID | Payload | Description |
| --- | --- | --- | --- |
| Start Flash OTA | 0x0001 | Bytes 2-5: Firmware size (uint32) | Initialize firmware update |
| Start SPIFFS OTA | 0x0004 | Bytes 2-5: Filesystem size (uint32) | Initialize SPIFFS update |
| Stop OTA | 0x0002 | None | Abort current update |
| Response | 0x0003 | Status code | Device response to commands |

Command Response Status Codes

| Status | Value | Description |
| --- | --- | --- |
| Accept | 0x0000 | Command accepted, OTA started |
| Reject | 0x0001 | Command rejected |
| Signature Error | 0x0003 | RSA signature verification failed |

Firmware Data Packet Format (App → Device)

Bytes 0-1: Sector Index (uint16, little-endian)
Byte 2: Packet Sequence (0x00-0xFE for data, 0xFF for final packet)
Bytes 3-N: Payload data (MTU - 4 bytes, typically up to 243 bytes)

Note: When Packet_Seq = 0xFF, the final 2 bytes contain CRC16 of the sector

Firmware ACK Format (Device → App via Notify)

Bytes 0-1: Sector Index (uint16, little-endian)
Bytes 2-3: ACK Status (uint16, little-endian)
Bytes 4-5: CRC16 checksum

Total: 6 bytes

ACK Status Codes

| Status | Value | Description |
| --- | --- | --- |
| Success | 0x0000 | Sector received successfully |
| CRC Error | 0x0001 | CRC verification failed |
| Sector Mismatch | 0x0002 | Unexpected sector index |
| Length Error | 0x0003 | Invalid payload length |
| OTA Start Failure | 0x0005 | Failed to initialize OTA |

OTA Update Flow

┌─────────────────────────────────────────────────────────────────┐
│                        OTA UPDATE FLOW                          │
├─────────────────────────────────────────────────────────────────┤
│                                                                 │
│  APP                                    DEVICE                  │
│   │                                        │                    │
│   │─── Write Command: Start OTA ──────────>│                    │
│   │    (0x0001, size=firmware_size)        │                    │
│   │                                        │                    │
│   │<── Notify Response: Accept ────────────│                    │
│   │    (0x0003, status=0x0000)             │                    │
│   │                                        │                    │
│   │    ┌─── FOR EACH SECTOR (4KB) ───────┐ │                    │
│   │    │                                 │ │                    │
│   │    │  Write FW Data: Packet 0        │ │                    │
│   │────│──(sector_idx, seq=0, data)──────│─>                    │
│   │    │                                 │ │                    │
│   │    │  Write FW Data: Packet 1...N-1  │ │                    │
│   │────│──(sector_idx, seq=1..N-1, data)─│─>                    │
│   │    │                                 │ │                    │
│   │    │  Write FW Data: Final Packet    │ │                    │
│   │────│──(sector_idx, seq=0xFF, CRC16)──│─>                    │
│   │    │                                 │ │                    │
│   │<───│── Notify ACK ───────────────────│─│                    │
│   │    │   (sector_idx, status, CRC)     │ │                    │
│   │    │                                 │ │                    │
│   │    └─────────────────────────────────┘ │                    │
│   │                                        │                    │
│   │    [All sectors transmitted]           │                    │
│   │                                        │                    │
│   │<── Device resets (500ms delay) ────────│                    │
│   │                                        │                    │
└─────────────────────────────────────────────────────────────────┘

Sector Transmission Details

| Parameter | Value |
| --- | --- |
| Sector Size | 4096 bytes (4KB) |
| Packet Size | MTU - 4 bytes (typically 243 bytes with MTU=247) |
| Packets per Sector | ~17 packets for 4KB sector |
| Sequence Numbers | 0x00 to 0xFE for data, 0xFF signals final packet |

Security Features

| Feature | Description |
| --- | --- |
| CRC16 | Per-sector integrity check |
| RSA-2048 | Firmware signature verification using SHA256 hash |
| Compression | Optional zlib compression for faster transfer (~30% improvement) |

Post-Update Behavior

After successful OTA completion:

- Device validates firmware signature (if enabled)

- Device waits 500ms (configurable)

- Device resets and boots new firmware

Error Recovery

| Scenario | Recovery |
| --- | --- |
| CRC Error | Retransmit current sector |
| Timeout | Restart from last ACK'd sector |
| Signature Fail | OTA rejected, device unchanged |
| Connection Lost | Resume from last successful sector |

Example: Calculate Transfer Time

Firmware size: 1MB (1,048,576 bytes)
Sector size: 4KB
Total sectors: 256
Packets per sector: ~17
Connection interval: 30ms
Packets per interval: 6 (conservative)

Time per sector: 17 / 6 * 30ms ≈ 85ms
Total time: 256 * 85ms ≈ 22 seconds

With zlib compression (~30% reduction):
Compressed size: ~734KB
Total time: ≈ 15 seconds

3. Time Sync Service

Service UUID:0x1802

Synchronizes device clock with mobile app/server time.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Current Time | 0x2C01 | Read, Write | Device current time |
| Time Zone | 0x2C02 | Read, Write | Timezone offset |
| DST Offset | 0x2C03 | Read, Write | Daylight saving offset |
| Last Sync | 0x2C04 | Read | Timestamp of last sync |

Current Time Format

Bytes 0-3: Unix timestamp (uint32, little-endian, seconds since 1970)
Bytes 4-5: Milliseconds (uint16, little-endian)

Time Zone Format

Byte 0: Timezone offset in 15-minute increments (int8)
        Example: UTC+2 = 0x08 (8 * 15 = 120 minutes)

DST Offset Format

Byte 0: DST offset in minutes (uint8)
        0x00 = No DST
        0x3C = 60 minutes DST

4. Terminal Command Service

Service UUID:0x1803

Provides debug and diagnostic terminal access.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| TX (Device→App) | 0x2D01 | Notify | Terminal output from device |
| RX (App→Device) | 0x2D02 | Write | Terminal input to device |
| Command Status | 0x2D03 | Read, Notify | Command execution status |

Command Format

Bytes 0-N: ASCII command string (null-terminated, max 200 bytes)

Response Format

Byte 0: Response type
  0x00 = Output line
  0x01 = Command complete (success)
  0x02 = Command complete (error)
  0x03 = Command in progress
Bytes 1-N: ASCII response data

Common Terminal Commands

| Command | Description |
| --- | --- |
| help | List available commands |
| status | Device status summary |
| reset | Soft reset device |
| factory | Factory reset (requires confirmation) |
| log [level] | Set/get log level |
| diag | Run diagnostics |

5. Device Sensors Service

Service UUID:0x1804

Monitors device sensor status and readings.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Sensor Status | 0x2E01 | Read, Notify | Combined sensor status bitmap |
| Temperature | 0x2E02 | Read, Notify | Internal temperature |
| Humidity | 0x2E03 | Read, Notify | Internal humidity |
| Motor Status | 0x2E04 | Read, Notify | Motor position and status |
| Lid Status | 0x2E05 | Read, Notify | Lid open/closed status |
| Pill Detection | 0x2E06 | Read, Notify | Pill presence sensors |

Sensor Status Bitmap

Byte 0: Status flags
  Bit 0: Temperature sensor OK
  Bit 1: Humidity sensor OK
  Bit 2: Motor encoder OK
  Bit 3: Lid sensor OK
  Bit 4: Pill sensors OK
  Bit 5: Battery sensor OK
  Bit 6-7: Reserved
Byte 1: Error flags
  Bit 0: Temperature out of range
  Bit 1: Humidity out of range
  Bit 2: Motor fault
  Bit 3: Lid jam detected
  Bit 4: Pill sensor fault
  Bit 5-7: Reserved

Temperature Format

Bytes 0-1: Temperature in 0.01°C (int16, little-endian)
           Example: 2350 = 23.50°C

Motor Status Format

Byte 0: Current position (0-11 for 12 slots)
Byte 1: Target position
Byte 2: Motor state
  0x00 = Idle
  0x01 = Moving
  0x02 = Homing
  0x03 = Error
Byte 3: Error code (if state = 0x03)

Pill Detection Format

Bytes 0-1: Pill presence bitmap (uint16)
           Bit N = 1 if pill present in slot N
           Example: 0x0FFF = all 12 slots have pills

6. Device Control Service

Service UUID:0x1805

Controls device movement and operations.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Control Command | 0x2F01 | Write, Indicate | Issue control commands |
| Control Status | 0x2F02 | Read, Notify | Current operation status |
| Position | 0x2F03 | Read, Write, Notify | Current/target position |

Control Commands

| Command | Value | Parameters | Description |
| --- | --- | --- | --- |
| Home | 0x01 | None | Return to home position |
| Rotate | 0x02 | Position (1 byte) | Rotate to position 0-11 |
| Extract | 0x03 | Position (1 byte) | Extract pill at position |
| Stop | 0x04 | None | Emergency stop |
| Calibrate | 0x05 | None | Run calibration sequence |

Command Request Format

Byte 0: Command code
Bytes 1-N: Command parameters (varies by command)

Command Response Format (Indicate)

Byte 0: Original command code
Byte 1: Result code
  0x00 = Success
  0x01 = Invalid command
  0x02 = Invalid parameter
  0x03 = Busy
  0x04 = Hardware error
  0x05 = Timeout
Bytes 2-N: Result data (varies by command)

Control Status Format

Byte 0: Device state
  0x00 = Idle
  0x01 = Executing command
  0x02 = Error state
  0x03 = Calibrating
Byte 1: Last command code
Byte 2: Last command result
Byte 3: Current position (0-11)

7. Extract Pill Service

Service UUID:0x1806

Manages pill extraction operations.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Extract Command | 0x3001 | Write, Indicate | Initiate extraction |
| Extract Status | 0x3002 | Read, Notify | Extraction operation status |
| Extract History | 0x3003 | Read | Recent extraction records |
| Schedule | 0x3004 | Read, Write | Scheduled extractions |

Extract Command Format

Byte 0: Command type
  0x01 = Extract from position
  0x02 = Extract next scheduled
  0x03 = Cancel extraction
Byte 1: Position (0-11, for type 0x01)
Byte 2: Confirmation flag (0x5A = confirmed)

Extract Status Format

Byte 0: Extraction state
  0x00 = Idle
  0x01 = Rotating to position
  0x02 = Extracting
  0x03 = Verifying
  0x04 = Complete (success)
  0x05 = Complete (failed)
Byte 1: Current position
Byte 2: Target position
Byte 3: Error code (if failed)
  0x00 = No error
  0x01 = No pill detected
  0x02 = Jam detected
  0x03 = Motor error
  0x04 = Timeout

Extract History Format

Each record (8 bytes):
  Bytes 0-3: Timestamp (uint32, Unix time)
  Byte 4: Position extracted
  Byte 5: Result (0x00 = success, other = error code)
  Bytes 6-7: Reserved

Max 10 records returned

Schedule Format

Each schedule entry (6 bytes):
  Byte 0: Day mask (bit 0 = Sunday, bit 6 = Saturday)
  Byte 1: Hour (0-23)
  Byte 2: Minute (0-59)
  Byte 3: Position (0-11)
  Byte 4: Enabled flag (0x00 = disabled, 0x01 = enabled)
  Byte 5: Reserved

Max 14 schedule entries

8. Sub Drive Service

Service UUID:0x1807

Provides sub drive (cartridge) information.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Sub Drive Info | 0x3101 | Read, Notify | Cartridge identification |
| Sub Drive Status | 0x3102 | Read, Notify | Cartridge status |
| Pill Count | 0x3103 | Read, Notify | Pills remaining per slot |
| Medication Info | 0x3104 | Read | Medication metadata |

Sub Drive Info Format

Bytes 0-15: Cartridge UUID (128-bit)
Bytes 16-19: Manufacturing date (uint32, Unix time)
Bytes 20-23: Expiration date (uint32, Unix time)
Byte 24: Total slots (typically 12)
Byte 25: Cartridge type
  0x01 = Standard
  0x02 = Extended
  0x03 = Custom

Sub Drive Status Format

Byte 0: Status
  0x00 = Not inserted
  0x01 = Inserted, valid
  0x02 = Inserted, expired
  0x03 = Inserted, error
Byte 1: Error code (if status = 0x03)
Bytes 2-3: Total extractions (uint16)
Bytes 4-5: Successful extractions (uint16)

Pill Count Format

Bytes 0-11: Pill count per slot (1 byte each, 0-255)
            Slot 0 at byte 0, slot 11 at byte 11
Byte 12: Total pills remaining

Medication Info Format

Bytes 0-31: Medication name (UTF-8, null-terminated)
Bytes 32-47: NDC code (ASCII, null-terminated)
Bytes 48-51: Dosage (uint32, micrograms)
Byte 52: Dosage unit
  0x01 = mg
  0x02 = mcg
  0x03 = IU

9. Notifications Service

Service UUID:0x1808

Handles event notifications and alerts.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Notification | 0x3201 | Notify | Push notifications to app |
| Notification Config | 0x3202 | Read, Write | Notification settings |
| Alert Status | 0x3203 | Read, Notify | Active alert status |
| Acknowledge | 0x3204 | Write | Acknowledge alerts |

Notification Format

Byte 0: Notification type
  0x01 = Scheduled extraction reminder
  0x02 = Extraction complete
  0x03 = Extraction failed
  0x04 = Low battery
  0x05 = Cartridge empty
  0x06 = Cartridge expiring
  0x07 = Device error
  0x08 = Connectivity lost
Bytes 1-4: Timestamp (uint32, Unix time)
Byte 5: Priority (0x01 = low, 0x02 = normal, 0x03 = high)
Bytes 6-N: Payload (varies by type, max 100 bytes)

Notification Config Format

Byte 0: Enabled notifications bitmap
  Bit 0: Scheduled reminders
  Bit 1: Extraction events
  Bit 2: Battery alerts
  Bit 3: Cartridge alerts
  Bit 4: Error alerts
  Bit 5-7: Reserved
Byte 1: Reminder lead time (minutes before scheduled)
Byte 2: Low battery threshold (percentage)
Byte 3: Quiet hours start (hour, 0-23)
Byte 4: Quiet hours end (hour, 0-23)

Alert Status Format

Byte 0: Active alerts bitmap
  Bit 0: Low battery
  Bit 1: Empty slot
  Bit 2: Expired cartridge
  Bit 3: Motor error
  Bit 4: Sensor error
  Bit 5: Missed dose
  Bit 6-7: Reserved
Bytes 1-4: Oldest alert timestamp

Acknowledge Format

Byte 0: Alert type to acknowledge (from alert bitmap)
        0xFF = Acknowledge all

10. Offline Sync Service

Service UUID:0x1809

Synchronizes offline data (sub drive logs, telemetry).

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Sync Control | 0x3301 | Write, Indicate | Control sync operations |
| Sync Data | 0x3302 | Notify | Streamed sync data |
| Sync Status | 0x3303 | Read, Notify | Sync progress status |
| Last Sync Time | 0x3304 | Read | Last successful sync timestamp |

Sync Control Commands

| Command | Value | Parameters | Description |
| --- | --- | --- | --- |
| Start Sync | 0x01 | Start timestamp (4 bytes) | Begin sync from timestamp |
| Pause Sync | 0x02 | None | Pause current sync |
| Resume Sync | 0x03 | None | Resume paused sync |
| Abort Sync | 0x04 | None | Cancel sync operation |
| Mark Complete | 0x05 | End timestamp (4 bytes) | Mark sync complete |

Sync Data Format

Byte 0: Record type
  0x01 = Extraction event
  0x02 = Error event
  0x03 = Telemetry sample
  0x04 = State change
Bytes 1-4: Timestamp (uint32, Unix time)
Byte 5: Sequence number (for ordering)
Bytes 6-N: Record payload (varies by type)

Extraction Event Payload

Byte 0: Position
Byte 1: Result (0x00 = success, other = error)
Byte 2: Duration (seconds)
Byte 3: Battery level at time

Telemetry Sample Payload

Bytes 0-1: Temperature (int16, 0.01°C)
Bytes 2-3: Humidity (uint16, 0.01%)
Byte 4: Battery level
Byte 5: Motor status

Sync Status Format

Byte 0: Sync state
  0x00 = Idle
  0x01 = Syncing
  0x02 = Paused
  0x03 = Error
Bytes 1-2: Records pending (uint16)
Bytes 3-4: Records synced (uint16)
Byte 5: Progress percentage (0-100)

11. Sound File Service

Service UUID:0x180B

Manages audio files for alerts and notifications.

Characteristics

| Characteristic | UUID | Properties | Description |
| --- | --- | --- | --- |
| Sound Control | 0x3401 | Write, Indicate | Sound playback control |
| Sound Upload | 0x3402 | Write Without Response | Upload sound data |
| Sound List | 0x3403 | Read | List available sounds |
| Sound Config | 0x3404 | Read, Write | Sound configuration |
| Volume | 0x3405 | Read, Write | Master volume level |

Sound Control Commands

| Command | Value | Parameters | Description |
| --- | --- | --- | --- |
| Play | 0x01 | Sound ID (1 byte) | Play sound by ID |
| Stop | 0x02 | None | Stop current playback |
| Upload Start | 0x03 | Sound ID, Size (4 bytes) | Begin upload |
| Upload End | 0x04 | Checksum (4 bytes) | Complete upload |
| Delete | 0x05 | Sound ID (1 byte) | Delete sound file |

Sound Upload Format

Bytes 0-1: Packet sequence (uint16)
Bytes 2-N: PCM audio data (8kHz, 8-bit mono, max 244 bytes)

Sound List Format

Each entry (20 bytes):
  Byte 0: Sound ID
  Bytes 1-16: Sound name (UTF-8, null-terminated)
  Bytes 17-18: Duration (uint16, milliseconds)
  Byte 19: Type
    0x01 = System
    0x02 = Alert
    0x03 = Custom

Sound Config Format

Byte 0: Reminder sound ID
Byte 1: Success sound ID
Byte 2: Error sound ID
Byte 3: Low battery sound ID
Byte 4: Extraction sound ID

Volume Format

Byte 0: Volume level (0-100)
Byte 1: Mute flag (0x00 = not muted, 0x01 = muted)

Error Codes Reference

Global Error Codes

| Code | Name | Description |
| --- | --- | --- |
| 0x00 | SUCCESS | Operation completed successfully |
| 0x01 | ERR_INVALID_PARAM | Invalid parameter value |
| 0x02 | ERR_INVALID_STATE | Operation not valid in current state |
| 0x03 | ERR_BUSY | Device busy with another operation |
| 0x04 | ERR_TIMEOUT | Operation timed out |
| 0x05 | ERR_NOT_FOUND | Requested resource not found |
| 0x06 | ERR_NO_MEMORY | Insufficient memory |
| 0x07 | ERR_HARDWARE | Hardware fault detected |
| 0x08 | ERR_COMM | Communication error |
| 0x09 | ERR_AUTH | Authentication required/failed |
| 0x0A | ERR_NOT_SUPPORTED | Feature not supported |
| 0xFF | ERR_UNKNOWN | Unknown error |

Connection Parameters

Recommended BLE Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| Connection Interval | 30-50 ms | Balance between latency and power |
| Slave Latency | 0-4 | Allow skipping connection events |
| Supervision Timeout | 4000 ms | Connection timeout |
| MTU | 247 bytes | Maximum transmission unit |

Advertising Parameters

| Parameter | Value |
| --- | --- |
| Advertising Interval | 100-200 ms (fast), 1000-2000 ms (slow) |
| Advertising Type | Connectable, Scannable |
| TX Power | 0 dBm |

Service Discovery

The device advertises the following service UUIDs:

- Device Information Service (0x180A)

- BLE OTA Service (00008018-0000-1000-8000-00805f9b34fb)

- Device Control Service (0x1805)

Full service list available after connection via GATT discovery.

Security

Pairing Requirements

- Bonding: Required

- MITM Protection: Required

- Secure Connections: Supported (LE Secure Connections)

- IO Capability: Display + Yes/No

Encryption

All characteristics except Device Information Service (read-only info) require an encrypted connection.

Authentication

Sensitive operations (FW Update, Factory Reset, Extract) require:

- Encrypted connection

- Bonded device

- Optional: PIN confirmation for critical operations

Version History

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | TBD | Initial specification |